A  Controlling Motors

Task 1: Interfacing of Servo Motor to Arduino

Yellow : data ( Pin 5)

Red : Vcc

Black / Brown : Gnd

Code for Servo Motor

#include <Servo.h>

Servo servo;

int angle = 10;

void setup() {

servo.attach(5);

servo.write(angle);

}

void loop()

{

// scan from 0 to 180 degrees

for(angle = 20; angle < 130; angle++)

{

servo.write(angle);

delay(15);

}

// now scan back from 180 to 0 degrees

for(angle = 130; angle > 10; angle--)

{

servo.write(angle);

delay(15);

}

}

Task 2 : DC Motor Speed and Direction Control

Try Running Motor in Forward and Reverse Direction

Control Speed of Motor

Simulate given Code  in TinkerCAD

L 293 D Motor Driver IC

Pin Enable 1: Pin 11 of Arduino ( Analog Pin)

Input 1 : Pin 6 of Arduino

Input 2: Pin 7 of Arduino

Output 1: Motor

Output 2: Motor

Gnd

Vcc

Code for DC Motor Control

void setup()

{

pinMode(11, OUTPUT);// En of Motor 1

pinMode(7,OUTPUT); // In1 of Motor 1

pinMode(6,OUTPUT);// In 2 of Motor 1

}

void fwdrev() // function reverses direction of motor

{

digitalWrite(7,LOW);// In1 = 0, FWD

digitalWrite(6,HIGH);//In2 =1

digitalWrite(11,HIGH);

delay(5000); // Wait for 5 second(s)

digitalWrite(7, LOW);// In1 = 0, Stop

digitalWrite(6,LOW);// In2 = 0

delay(5000); // Wait for 1000 millisecond(s)

digitalWrite(7, HIGH);//In1 =1, REV

digitalWrite(6,LOW);//In2 =0

delay(5000);

digitalWrite(7, LOW);// In1 = 0, Stop

digitalWrite(6,LOW);// In2 = 0

}

void speed () // function for 2 speed settings of DC Motor

{

digitalWrite(7, LOW);

digitalWrite(6,HIGH);

analogWrite(11,50); // Slow Speed

delay(25000); // Wait for 1000 millisecond(s)

analogWrite(11,200);// High Speed

digitalWrite(7, LOW);

digitalWrite(6,HIGH);

delay(25000); // Wait for 1000 millisecond(s)

}

void loop()

{

fwdrev();

speed();

}

1.         Interfacing DC Motor with Arduino

// Motor A connections

int enA = 9;

int in1 = 13;

int in2 = 12;

// Motor B connections

int enB = 3;

int in3 = 11;

int in4 = 10;

void setup() {

pinMode(enA, OUTPUT);

pinMode(enB, OUTPUT);

pinMode(in1, OUTPUT);

pinMode(in2, OUTPUT);

pinMode(in3, OUTPUT);

pinMode(in4, OUTPUT);

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, LOW);

}

void loop() {

directionControl();

delay(1000);

speedControl();

delay(1000);

}

void directionControl() {

// Set motors to maximum speed

// PWM value ranges from 0 to 255

analogWrite(enA, 255);

analogWrite(enB, 255);

// Turn on motor A & B

digitalWrite(in1, HIGH);

digitalWrite(in2, LOW);

digitalWrite(in3, HIGH);

digitalWrite(in4, LOW);

delay(2000);

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

digitalWrite(in3, LOW);

digitalWrite(in4, HIGH);

delay(2000);

// Turn off motors

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, LOW);

}

void speedControl() {

// Turn on motors

digitalWrite(in1, LOW);

digitalWrite(in2, HIGH);

digitalWrite(in3, LOW);

digitalWrite(in4, HIGH);

for (int i = 0; i < 256; i++) {

analogWrite(enA, i);

analogWrite(enB, i);

delay(20);

}

for (int i = 255; i >= 0; --i) {

analogWrite(enA, i);

analogWrite(enB, i);

delay(20);

}

// Now turn off motors

digitalWrite(in1, LOW);

digitalWrite(in2, LOW);

digitalWrite(in3, LOW);

digitalWrite(in4, LOW);

}