# Name: Dhairya Shah

# Roll no.: 16010324044

# Batch: A3

# Time Division Multiplexing (TDM):

## Theory:

Time Division Multiplexing (TDM) is a technique in which multiple signals share a single communication channel by transmitting them in different time slots. Each signal occupies the channel for a short duration in a cyclic manner. At the receiver, synchronization is used to separate and reconstruct the original signals. TDM is widely used in digital communication systems.

## Code:

clc; %clear command window

clear all; %clear variables

close all; %close all open variables

t = 0:0.001:1; %time interval

% Original signals

x1 = sin(2*pi*5*t); %sine wave

x2 = square(2*pi*3*t); %square wave

% TDM Multiplexing

tdm_signal = zeros(1, length(t)*2); %initialize

tdm_signal(1:2:end) = x1; %time slot 1

tdm_signal(2:2:end) = x2; %time slot 2

% Plot original signals

figure;

subplot(2,1,1);

plot(t,x1); title('Original Signal 1 (Sine Wave)');

subplot(2,1,2);

plot(t,x2); title('Original Signal 2 (Square Wave)');

% Plot multiplexed signal

figure;

stem(tdm_signal(1:1000)); %discrete representation of 'plot'

title('TDM Signal');

% DEMULTIPLEXING

y1 = tdm_signal(1:2:end); %recover x1

y2 = tdm_signal(2:2:end); %recover x2

% Plot recovered signals

figure;

subplot(2,1,1);

plot(t,y1); title('Recovered Signal 1');

subplot(2,1,2);

plot(t,y2); title('Recovered Signal 2');

## Output:

# Frequency Division Multiplexing (FDM):

## Theory:

Frequency Division Multiplexing (FDM) is a technique where multiple signals are transmitted simultaneously by assigning different frequency bands to each signal. Each signal modulates a different carrier frequency, and all signals are combined for transmission. At the receiver, filters and demodulation are used to recover the original signals.

## Code:

clc;

clear all;

close all;

t = 0:0.001:1; %time

% Original signals

m1 = sin(2*pi*5*t); %signal 1

m2 = square(2*pi*3*t); %signal 2

% Carrier signals

c1 = cos(2*pi*50*t);

c2 = cos(2*pi*100*t);

% FDM Multiplexing

s1 = m1 .* c1;

s2 = m2 .* c2;

fdm_signal = s1 + s2;

% Plot original signals

figure;

subplot(2,1,1);

plot(t,m1); title('Original Signal 1');

subplot(2,1,2);

plot(t,m2); title('Original Signal 2');

% Plot multiplexed signal

figure;

plot(t,fdm_signal);

title('FDM Signal');

% DEMODULATION

d1 = fdm_signal .* c1;

d2 = fdm_signal .* c2;

% LOW PASS FILTER

y1 = lowpass(d1,10,1000);

y2 = lowpass(d2,10,1000);

% Plot recovered signals

figure;

subplot(2,1,1);

plot(t,y1); title('Recovered Signal 1');

subplot(2,1,2);

plot(t,y2); title('Recovered Signal 2');

## Output:

# Post Lab Activity:

## Code:

clc;

clear all;

close all;

t = 0:0.001:1;

x1 = sin(2*pi*5*t);

x2 = sin(2*pi*10*t);

x3 = sin(2*pi*15*t);

tdm_signal = zeros(1,length(t)*3);

tdm_signal(1:3:end) = x1;

tdm_signal(2:3:end) = x2;

tdm_signal(3:3:end) = x3;

% Plot multiplexed

figure;

stem(tdm_signal(1:1500));

title('TDM with 3 Signals');

% DEMULTIPLEXING

y1 = tdm_signal(1:3:end);

y2 = tdm_signal(2:3:end);

y3 = tdm_signal(3:3:end);

% Plot recovered

figure;

subplot(3,1,1); plot(t,y1); title('Recovered Signal 1');

subplot(3,1,2); plot(t,y2); title('Recovered Signal 2');

subplot(3,1,3); plot(t,y3); title('Recovered Signal 3');

## Output:

## Code:

clc;

clear all;

close all;

t = 0:0.001:1;

m1 = sin(2*pi*5*t);

m2 = sin(2*pi*10*t);

m3 = sin(2*pi*15*t);

c1 = cos(2*pi*50*t);

c2 = cos(2*pi*100*t);

c3 = cos(2*pi*150*t);

s1 = m1 .* c1;

s2 = m2 .* c2;

s3 = m3 .* c3;

fdm_signal = s1 + s2 + s3;

% Plot multiplexed

figure;

plot(t,fdm_signal);

title('FDM with 3 Signals');

% DEMODULATION

d1 = fdm_signal .* c1;

d2 = fdm_signal .* c2;

d3 = fdm_signal .* c3;

% LOW PASS FILTER

y1 = lowpass(d1,10,1000);

y2 = lowpass(d2,10,1000);

y3 = lowpass(d3,10,1000);

% Plot recovered

figure;

subplot(3,1,1); plot(t,y1); title('Recovered Signal 1');

subplot(3,1,2); plot(t,y2); title('Recovered Signal 2');

subplot(3,1,3); plot(t,y3); title('Recovered Signal 3');

## Output:

## Applications:

TDM:

- Digital telephony

- Data communication systems

- Satellite communication

FDM:

- Radio broadcasting

- Television transmission

- Cable TV

# Conclusion:

In this experiment, I learnt how to implement TDM and FDM using MATLAB. TDM multiplexes signals in different time slots, while FDM uses different carrier frequencies. The original signals were successfully recovered using demultiplexing techniques.