| Course Name: | Basic of Electrical Engineering Laboratory 316U06L102 | Semester: | I |
| --- | --- | --- | --- |
| Date of Performance: | / / 20-- | Batch No: |  |
| Student Name: |  | Roll No: |  |
| Faculty Sign & Date: |  | Grade/Marks: | / 20 |

Experiment No: 7

Title: Three phase Measurement of Power using Two Wattmeter Method

Aim and Objective of the Experiment:

- To measure the power of three phase power using Two Wattmeter Method

COs to be achieved:

CO2: Demonstrate and analyze steady state response of single phase and three phase circuits

Circuit Diagram:

Stepwise-Procedure:

- 1. Connect the circuit as shown in circuit diagram - 2. Increase the load and note down the reading VL,IL,W1 and W2 - 3. Practically you will obtain total power W=W1+W2 - 4. Theoretically power is measured by using formula P=√3VLILcosϕ, using cosϕ=1(unity) for resistive load.

Observation Table: Sr.no; VL (Volts); IL (Amp); W1 (KW); W2 (KW); W= (W1+W2) (KW); P = √3VLILCOSϕ (KW); Lamp load given from lamp bank (KW). TH; PR; TH; PR; TH; PR; TH; PR. 1. 2. 3. 4. Theoretical Calculations: Power= √3 x VL x IL x cos φ cos φ=1 Power =Wattage rating of lamp load x No of lamps (One lamp is of 100W rating) W1= VL x IL x cos (30+φ) Φ=0 W2= VL x IL x cos (30-φ) Total Power=P=W1+W2

Conclusion:

Signature of faculty in-charge with Date: