Batch: A3	Roll No.: 16010324044

Experiment No. 6

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

TITLE: Block Addition

AIM: Write an Assembly Language Program for 8051 to add the block of 5 numbers stored in consecutive memory locations and store the result in the memory.

OUTCOME:

Student will be able to develop programming skills using instruction set of microprocessor 8051

Assumptions about data:

Before Execution

| Memory Location Address | Data |
| --- | --- |
| 30H | 11H |
| 31H | 12H |
| 32H | 13H |
| 33H | 14H |
| 34H | 15H |
| 35H | 16H |
| 36H | 17H |

Program:

| Instructions/Mnemonics | Comments |
| --- | --- |
| MOV R0, #30H | Load starting address of data block |
| MOV R1, #07H | Load counter with number of data bytes (7) |
| MOV R2, #00H | Clear high byte of result |
| CLR A | Clear accumulator (low byte of sum) |
| GO: ADD A, @R0 | Add data at memory location pointed by R0 |
| JNC NEXT | If no carry, jump to NEXT |
| INC R2 | If carry, increment high byte |
| NEXT: INC R0 | Move to next memory location |
| DJNZ R1, GO | Decrement counter and repeat until zero |
| MOV 40H, A | Store result low byte at 40H |
| MOV 41H, R2 | Store result high byte at 41H |
| END | End of program |

Result:

| Memory Location Address | Data |
| --- | --- |
| 40H | 8CH |
| 41H | 00H |

Conclusion: (specify the concepts learnt in this experiment)

In this experiment, I learnt how to do block addition using the 8051 microcontroller. It also gave me a better understanding of indirect addressing (@R0), loop control using DJNZ, and carry management

with JNC.

Signature of faculty in-charge