| K. J. Somaiya College of Engineering, Mumbai-77 (Autonomous College Affiliated to University of Mumbai) Semester: July – November 2018 Max. Marks: 30 Duration: 1hr.15 min. Class: SY B. Tech Semester: III Branch: EXTC Test 1 Full name of the course : Digital Logic Design Course Code:UETC303 |  |  |  |
| --- | --- | --- | --- |
| Q. No | Questions | Marks | CO Mapped |
| Q1 | Use Quine – Mc Cluskey to minimize the following expression F(ABCD) = ∑m(0, 2, 4, 6,7,8,9,10,13) OR If Y(A,B,C,D)= ABC +D - Draw the corresponding K map represented by above expression - Write canonical SOP form of expression - Realise using NAND gates - Write minimized POS expression corresponding to the same | 10 | CO2 |
| Q2 | Design a full subtractor using suitable decoder. | 10 | CO2 |
| Q3A | Perform the following. Show your calculations - Convert (A20)16 into decimal. - (18.4)10 into hexadecimal. - (543)10 into BCD - (78)10-(53)10 using two’s compliment method | 05 | CO1 |
| Q3B | Write Truth table for following logic A security system is designed to switch on a powerful floodlight when movement is detected or a panic button is pressed. The system should operate only when it is dark. The specifications for the three input subsystems are given below: Light Sensor A Dark = Logic 0 Movement Sensor B Movement = Logic 1 Panic Button C Pressed = Logic 1 | 05 | CO1 |