| Course Name: Course Code: | Basic of Electrical Engineering BEEL) 316U06L102 | Semester: | I |
| --- | --- | --- | --- |
| Date of Performance: | / / 20-- | Batch No: |  |
| Student Name: |  | Roll No: |  |
| Faculty Sign & Date: |  | Grade/Marks: | / 20 |

Experiment No: 4

Title: Maximum Power Transfer Theorem

Aim and Objective of the Experiment:

- To observe maximum power transfer across load resistor in a D.C circuit.

COs to be achieved:

CO1: Analyze resistive networks excited by DC sources using various network theorems.

Circuit Diagram:

VS = 15 V and RS = _____ Ω

Stepwise-Procedure:

1. Set D.C. supply voltage VS = 15 V 2. Vary RL in the range 100 Ω - 1 KΩ in steps of 100 Ω 3. Note down IL and VL for each value of RL. Where IL and VL are current through RL and voltage across RL respectively. 4. Prepare observation table showing readings of RL Vs power P = IL . VL 5. Plot graph of P Vs RL 6. Locate the point of maximum value of power P and note down corresponding value of RL. Verify the results theoretically

Observation Table:

Sr. No.; RL Ω; Circuit Current (IL) in mA; Voltage (VL) in Volts; Power absorbed by load (PL) in W PL = IL2.RL. Theoretical; Practical; Theoretical; Practical. 100. 200. 300. 400. 500. 600. 700. 800. 900. 1 K.

Graph: Draw a graph showing effect of variation in RL on PL using observation table. Take RL on X –axis and PL on Y- axis. (Use a graph paper)

Conclusion- - Explore one practical application where Maximum Power Transfer Theorem is used. - Draw a block diagram or circuit diagram of this application. - Explain in brief. OR Answer the following: - Do you apply Thevenin’s Theorem to calculate Maximum Power across load resistor in a D.C. circuit? - Take a sample problem. Draw a block diagram or circuit diagram of this sample problem. - Explain the solution in brief.

Signature of faculty in-charge with Date: