3. The two blocks shown are originally at rest. Neglecting the masses of the pulleys and the effect of friction in the pulleys and between block A and the incline, determine (a) the acceleration of each block, (b) the tension in the cable.

4. The coefficients of friction between blocks A and C and the horizontal surfaces are μk = 0.2.Knowing that  mA = 10 kg, mB = 10 kg and  mC =5kg  determine (a) the tension in the cord, (b) the acceleration of each block.

5. The blocks A and B are connected by a cable that runs around two pulleys of negligible weight. Determine the time required for A to reach a speed of 2 m/s after the system has been released from rest. Neglect friction.

6. The 100-kg block A shown in Fig. is released from rest. If the masses of the pulleys and the cord are neglected, determine the speed of the 20-kg block B in 2 s.

A block of weight 4 KN is being pulled up the inclined plane by using another block B of weight 8 KN as shown in the figure. Determine (i) acceleration of block A, (ii) Acceleration of block B, (iii) Tension T1 in cord attached to block A. (iv) Tension T2 in cord attached to Block B. Take coefficient of friction between block A and inclined plane as 0.25. Assume cords as inextensible and pulleys at C and 0 are small, frictionless and negligible mass.

( Ans : aA = 2.137 m/s² , T = 3564.273 N )

The 100 kg crate is hoisted up by the incline using the cable and motor M. For a short time, the force in the cable is F= 800t2 N where t is in seconds. If the crate has an initial velocity V1= 2 m/s when t = 0 seconds determine the velocity when t =2 seconds. The coefficient of kinetic friction between the crate and incline is μk = 0.3.

( Ans : 8.91 m/s )

An air plane has a mass of 25000 kg and its engines develop a total thrust of 40 KN along the runway. The force of air resistance to motion of air plane is given by D = 2.25 V², where V is in m/s and D is in Newtons. Determine the length of runway required if the plane takes off and becomes airborne at a speed of 240 KPH.

( Ans : 1.598 KM )