Batch:     A3          Roll No.:     16010324044

Experiment / assignment / tutorial No._______

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

TITLE: Multiplexer and Decoder AIM: To study, design and implement logic functions using Multiplexer and Decoder OUTCOME: Design and implement combinational logic circuits and their applications

ICs used:

Procedure:

- Realize following combinational logic circuits using IC 74151 and logic             gates and verify the working by providing different combinations of inputs.

  - Using only one 8:1

- Realize following combinational logic circuits using IC 74138 and logic gates and verify the working by providing different combinations of inputs.

F1 (A, B, C,D) = Σ m ( 1, 4, 5, 6, 9, 11, 15 ) using only one 8:1multiplexer

Circuit diagram

Truth Table

| Y | D |  | 0 |  | 1 |  | D’ |  | D |  | D |  | 0 |  | D |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| A | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 | 0 | 1 |
| B | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| C | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| D | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

Pin diagram of  IC 74151

Pin diagram of  IC 74138

F2= Σ m ( 0, 2, 4, 6, 7  ) using IC 74138 decoder

Circuit diagram using decoder

Truth Table

| A | B | C | Y |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 1 |
| 0 | 0 | 1 | 0 |
| 0 | 1 | 0 | 1 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |

Conclusion:

We have successfully designed and implemented the specified function using an 8:1 multiplexer. Additionally, we have implemented another given function using a 3:8 decoder. Afterward, we documented the output obtained from both implementations and included the pin diagrams for both the IC 74151 (Multiplexer) and the IC 74138 (Decoder).

Roll No: 16010324044                                           Signature of faculty in-charge with date