| Batch: 44 | Roll No.: 16010324044 |
| --- | --- |
| Experiment No. 7 |  |
| Grade: AA / AB / BB / BC / CC / CD /DD |  |

Signature of the Staff In-charge with date

TITLE: Generation of Square wave using Timers of 8051

AIM: Write a program of 8051 to generate a square wave using on-chip Timers.

OUTCOME:

Students will be able to develop programming skills using instruction set of microcontroller 8051.

Diagram:

Timer 0  in Mode 1

TMOD: (Draw Control Word Format and Control Word obtained)

TCON: (Control Word Format and Control Word obtained)

Program:

CLR P1.0                                   ;

AGAIN:MOV TMOD,#01H     ; timer 0 in mode 1

MOV TL0,#0CH                       ;

MOV TH0,#0FEH                     ; load the timer with initial count

SETB TR0                                 ; START TIMER 0

HERE:JNB TF0 ,HERE             ;

CPL P1.0                                    ; COMPLIMENT P1.0

CLR TR0                                   ; STOP THE TIMER

CLR TF0                                     ; CLEAR THE TIMER FLAG

SJMP AGAIN

//square wave of 1 KHz,Timer0 in mode 1,Crystal freq=12MHz

Result (Waveform Obtained):

Conclusion: : (specify the concepts learnt in this experiment)

In this experiment, I learned how to use the 8051 timer in mode 1 for generating a square wave. I understood timer registers, control words, and port pin toggling, in assembly language programming.

Signature of faculty in-charge