K. J. Somaiya College of Engineering, Mumbai-77

(A Constituent College of Somaiya Vidyavihar University)

Batch: A3	Roll No.: 16010324044

Experiment No. 8

Grade: AA / AB / BB / BC / CC / CD /DD Signature of the Staff In-charge with date

TITLE: Interfacing of ADC with 8051

AIM: Design a ADC interface for 8051 and write a program to convert an analog signal to its equivalent digital value.

OUTCOME:

Students will be able to understand the interfacing of peripheral ICs with 8051 to design various applications.

Diagram of Interfacing of ADC:

K. J. Somaiya College of Engineering, Mumbai-77

(A Constituent College of Somaiya Vidyavihar University)

Explanation of ADC interfacing with 8051 (Communication between ADC and 8051):

- Configure Ports:

  - Set the data port (e.g., P1) as input to read ADC output.

  - Configure control lines (ALE, START, OE, EOC, ADDR_A/B/C for channel selection) on Port 3.

- Select ADC Channel (if applicable):

  - Use ADDR_A, ADDR_B, ADDR_C to select the desired channel for multi-channel ADCs.

- Start Conversion:

  - Set ALE high to latch the address.

  - Apply a START pulse (set START high, then low) to begin conversion.

- Monitor End of Conversion (EOC):

  - Poll the EOC pin; it goes high when conversion completes.

- Read Digital Data:

  - Set OE high to enable ADC output.

  - Read the converted digital data from the data port (P1).

- Reset for Next Conversion:

  - Clear OE and START to reset the ADC.

Program:

K. J. Somaiya College of Engineering, Mumbai-77

((A Constituent College of Somaiya Vidyavihar University)

| Mnemonic | Comment |
| --- | --- |
| ADDR_C BIT P3.0 | Define ADDR_C as bit P3.0 – used for addressing channel C on ADC |
| ADDR_B BIT P3.1 | Define ADDR_B as bit P3.1 – used for addressing channel B on ADC |
| ADDR_A BIT P3.3 | Define ADDR_A as bit P3.3 – used for addressing channel A on ADC |
| ALE BIT P3.4 | Define ALE as bit P3.4 – used as Address Latch Enable for ADC |
| OE BIT P3.5 | Define OE as bit P3.5 – used as Output Enable for ADC |
| START BIT P3.6 | Define START as bit P3.6 – used to start the ADC conversion |
| EOC BIT P3.7 | Define EOC as bit P3.7 – used to monitor End of Conversion from ADC |
| ORG 0000H | Set origin (starting address) of the program to 0000H |
| MOV P1, #0FFH | Make Port 1 an input port by writing all 1s |
| SETB EOC | Set EOC (End of Conversion) bit high initially |
| CLR ALE | Clear ALE to prepare for the next ADC operation |
| CLR START | Clear START to initialize the ADC |
| CLR OE | Clear OE (Output Enable) to disable ADC output initially |
| NEXT_SAMPLE: | Label for the start of a new ADC sample sequence |
| CLR ADDR_C | Clear ADDR_C to select channel 2 on the ADC |
| SETB ADDR_B | Set ADDR_B to select channel 2 on the ADC |
| CLR ADDR_A | Clear ADDR_A to select channel 2 on the ADC |
| NOP | No Operation – short delay for address setup |
| SETB ALE | Set ALE (Address Latch Enable) to latch the address bits into ADC |
| NOP | No Operation – short delay after setting ALE |
| SETB START | Apply a START pulse to initiate ADC conversion |
| NOP | No Operation – delay after starting ADC conversion |

K. J. Somaiya College of Engineering, Mumbai-77

(A Constituent College of Somaiya Vidyavihar University)

| CLR ALE | Clear ALE after starting the conversion |
| --- | --- |
| CLR START | Clear START to prepare for the next cycle |
| JB EOC, $ | Wait for EOC to go high (polling EOC) to check if conversion is complete |
| JNB EOC, $ | Keep waiting until EOC goes low, indicating the conversion is finished |
| SETB OE | Set OE to enable ADC output (ready to read the converted value) |
| HERE: SJMP HERE | Infinite loop to hold the program here after conversion |
| END | End of program |

Results:

| Analog Input | Digital Output |
| --- | --- |
| 0 | 00000000 |
| 0.5 | 00100000 |
| 1 | 00111111 |
| 1.5 | 01011100 |
| 2 | 01111011 |
| 2.5 | 10011010 |
| 3 | 10111001 |
| 3.5 | 11010010 |
| 4 | 11110001 |
| 4.27 | 11111111 |

K. J. Somaiya College of Engineering, Mumbai-77

(A Constituent College of Somaiya Vidyavihar University)

Conclusion: (specify the concepts learnt in this experiment)

In this experiment, I learnt ADC interfacing for 8051 and successfully converted analog signals to digital signals. I also learnt concepts like ADC, microcontroller port configuration, digital communication with ADC, timing and polling techniques.

Signature of faculty in-charge