Somaiya Vidyavihar University K J Somaiya School of Engineering

Batch: Roll No.:

Experiment / assignment / tutorial No. 07

Grade: AA / AB / BB / BC / CC / CD /DD

Signature of the Staff In-charge with date

Title: Pulse Position Modulation and Demodulation Objectives: 1. To understand generation process of PPM from PAM and PWM 2. To simulate PPM waveform for a given message sign a 3. To reconstruct the original signal through a demodulation process 4. To study the impact of modulation index and pulse width on the signal OUTCOME: 1. understand working of analog communication transmitter and receiver systems 2. compare Pulse modulation systems and multiplexing techniques. 3. understand noise effect on the performance of communication systems.

∙ Theory

o PPM is a derivative of PWM

o First PWM signal is generated by comparing message signal with a sawtooth  carrier. The trailing edges of the PWM pulses change position based on the

message.

∙ Design Parameters

o Message frequency (fm) = 2 Hz

o Carrier/sampling frequency (fs) = 1000 Hz

o Modulation index: controlled by the amplitude of the message signal

o Pulse width: Defined as a percentage of the sampling period

o Develop a logic for PPM demodulation (Integrator and envelope detector).  Embed in the same code and demodulate PPM waveform

o Generate another figure to show PWM, PPM and demodulated signal

∙ Design Parameters

o Open MATLAB Online.

o Develop logic to generate PPM generation and display the message signal,  carrier sawtooth signal, PWM signal, PPM signal.

o Observe the PPM plot: notice how the gap between pulses changes as the  message signal sine wave goes up and down.

o Change the pulse_width variable and observe the changes in the output.

Variations in Parameters

o Vary fc (Carrier Frequency): Increasing fc increases the number of pulses.  This results in a higher resolution and a more accurate reconstruction of the  message.

o Vary Message Amplitude: If the message amplitude exceeds the carrier  amplitude, over-modulation occurs, and pulses will be lost or merged.

Department of Electronics and Telecommunication Engineering (Programme: EXTC) SY B.Tech/EXTC/CS/Sem-IV/January-April 2026 Page 1 of 2

Somaiya Vidyavihar University K J Somaiya School of Engineering

∙ Post lab activity:

o Shift in the position of each pulse of PPM depends on what?

o State practical applications of PPM.

Signature of faculty in-charge

Department of Electronics and Telecommunication Engineering (Programme: EXTC) SY B.Tech/EXTC/CS/Sem-IV/January-April 2026 Page 2 of 2